18KU30A 2000 5650KW 50HZ X 2UNITS

| Price： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MAKER; MITSUBISHI. TYPE; 18KU30A. MFG YEAR; 2001. SPEC. OUTPUT; 5,650KW. REVOLUTION; 750 rpm. CYL. NO. ,; 18. BORE. STROKE; 380mm. FUEL; DIESEL. QTY; 2 UNIT. RUNNING HR; No.1 : 51,129HR No.2 : 48,000HR. LAST OVERHAUL; 2007,OCT,11. STOP ENGINE; 2008,JAN. |
| Engine Manufacturer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Years |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Model |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Year of Manufacture |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total Hours No. 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total Hours No. 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Rated Output in KW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RPM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Generator Manufacturer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Model |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Year of Manufacture |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total Hours No. 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total Hours No. 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output in Kw |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Output Voltage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Power Factor |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RPM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Weight: & Dimension |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| L = mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| H = mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R = mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Net weight kgs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NO. | Quantity |  | Weight Description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | Skid mounted Wartsila 18V26 @ 1000 rpm & ABB Alternator, 6150KVA,50 HZ, 6,300 Volts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  | Walkways for engine skids |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  | Exhaust System to Waste boilers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  | Waste heat boilers for steam supply for HFO heating |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  | Silencers for exhaust |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  | Plate heat exchangers for engine cooling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  | HFO Centrafuges for treatment of HFO fuel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  | Fuel transfer Pumps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  | HFO heating tank for Viscosity Control centre |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  | HFO Viscosity control Skid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  |  | All associated Pipe work |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  | Air intake pipe work System to Ar cleaners and Air Cleaners |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | Water treatment for Boilers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | Starter System Air Reciever |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  | Air Compressor for Air start System |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  | Engine top up Oil Storage tank and piping and valves and pump |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 |  |  | Gen Sync comtrol board |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 |  |  | Auxiliary Switch control boards |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 |  |  | HFO Day heated, insulated tanks & ass. pipe work, Pumps & valves |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 |  |  | Complete Fuel Delivery pump Station and all pumping and valve equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 21 |  |  | Cooling Towers and all Pumping and piping and valves |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22 |  |  | Overhead crane for maintaince |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 23 |  |  | Manuals, Installation Manual & operating manuals& spare parts manuals |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24 |  |  | Aprox US 150,000 worth of NEW spare operational Spares |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COMPLETE OPERATIONAL POWER PLANT FROM FUEL DELIVERY TO SYNC BOARD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.0 GENERAL MAIN DATA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.0.0 General: The power plant was designed, engineered and delivered by Stork Wartsila diesel. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.0.1 Type of power plant: The diesel generating sets were designed for heavy fuel oil operation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.0.2 Main data, conditions: The total electrical output is generated by two (2) diesel generating sets of type 18V26, at 1000RPM, with the following electrical characteristics. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Station nominal output (ISO) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Generator nominal output |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Voltage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Auxiliary voltage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Station actual output at site |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.1 DIESEL GENERATING SET A.1.1 Diesel engine： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Technical data： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Engine type: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of cylinders |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | in Vee |  |
| Bore |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mm |  |
| Stroke |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | mm |  |
| Maximum continuous rating |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | kWm |  |
| Speed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RPM |  |
| Direction of rotation facing flywheel side |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Clock wise |  |
| Fuel consumption. Based on fuels having a calorific value of 42,700 kJ/kG at 100% load 186g/kWh |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.1.2 Generator: (Design and construction) Two (2) three-phase, brushless self-regulating rotating field synchronous generator with AC-exciter and rectifier dics in line incorporated electronic voltage regulator and equipped with damper wading, suitable formutual parallel operation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Technical data: |  |  |  |  |  |  |  |  |
| Rating (at site conditions) |  |  |  |  |  |  |  |  | kVA |  |  |  |  |  |  |  |  |
| Output |  |  |  |  |  |  |  |  | kW |  |  |  |  |  |  |  |  |
| Efficiency |  |  |  |  |  |  |  |  | % |  |  |  |  |  |  |  |  |
| Power factor |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Voltage: frequency |  |  |  |  |  |  |  |  | Volts/50 Hz |  |  |  |  |  |  |  |  |
| Speed |  |  |  |  |  |  |  |  | RPM |  |  |  |  |  |  |  |  |
| A.2 AUXILIARY SYSTEMS A.2.1 Fuel system |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.2.1.0 Type of fuel: Heavy fuel oil 380 cst (50°C) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.2.1.1 Heavy fuel oil system: The heavy fuel oil is transferred from a storage tank by the transfer pumps to the settling tank. From the settling tank the fuel oil is pumped to the separator unit where it is purified. After purification the fuel is transferred to day tank and then supply to engine for operation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SYSTEM COMPOSITION. A.2.1.1.0 heavy fuel oil unloading module(U691), including |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Electrical motor driven pump |  |  |  |  |  |  |  |  |  |  |  |  |  |  | set |  |  |
| Flow rate: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | M3/h |  |  |
| Electrical motor, each |  |  |  |  |  |  |  |  |  |  |  |  |  |  | kW |  |  |
| Pressure head |  |  |  |  |  |  |  |  |  |  |  |  |  |  | bar |  |  |
| Control panel |  |  |  |  |  |  |  |  |  |  |  |  |  |  | set |  |  |
| Common base frame of steel |  |  |  |  |  |  |  |  |  |  |  |  |  |  | set |  |  |
| Valve and connection pipe |  |  |  |  |  |  |  |  |  |  |  |  |  |  | set |  |  |
| A.2.1.1.1 H.F.O. transfer module (U692), including |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Electrical motor driven pump |  |  |  |  |  |  |  |  |  |  |  |  |  | set |  |  |  |
| Flow rate |  |  |  |  |  |  |  |  |  |  |  |  |  | M3/h |  |  |  |
| Electrical motor, each |  |  |  |  |  |  |  |  |  |  |  |  |  | kW |  |  |  |
| Pressure head |  |  |  |  |  |  |  |  |  |  |  |  |  | bar |  |  |  |
| Control Panel |  |  |  |  |  |  |  |  |  |  |  |  |  | set |  |  |  |
| Common base frame of steel |  |  |  |  |  |  |  |  |  |  |  |  |  | set |  |  |  |
| Valve and connection pipe |  |  |  |  |  |  |  |  |  |  |  |  |  | set |  |  |  |
| A.2.1.1.2 H.F.O. buffer tank (V 673), including: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Capacity |  |  |  |  |  |  |  |  |  |  |  | M3 |  |  |  |  |  |
| High and low level alarm |  |  |  |  |  |  |  |  |  |  |  | set |  |  |  |  |  |
| Locally mounted level indicator |  |  |  |  |  |  |  |  |  |  |  | set |  |  |  |  |  |
| Steam heater 7 bars |  |  |  |  |  |  |  |  |  |  |  | set |  |  |  |  |  |
| Type of tank |  |  |  |  |  |  |  |  |  |  |  | Horizontal |  |  |  |  |  |
| A.2.1.1.3 H.F.O. service tank (V 671), including |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Capacity |  |  |  |  |  |  |  |  |  |  |  | M3 |  |  |  |  |  |
| High and low level alarm |  |  |  |  |  |  |  |  |  |  |  | set |  |  |  |  |  |
| Locally mounted level indicator |  |  |  |  |  |  |  |  |  |  |  | set |  |  |  |  |  |
| Steam heater 7 bars |  |  |  |  |  |  |  |  |  |  |  | set |  |  |  |  |  |
| Type of tank |  |  |  |  |  |  |  |  |  |  |  | Horizontal |  |  |  |  |  |
| A.2.1.2 Light fuel oil system: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The light fuel oil is unloaded from the truck to storage tank and suply to the engine for cold starting condition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SYSTEM COMPOSITION. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.2.1.2.0 light fuel oil unloading module(U631), including |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Electrical motor driven pump |  |  |  |  |  |  |  |  |  |  |  |  |  | set |  |  |  |
| Flow rate |  |  |  |  |  |  |  |  |  |  |  |  |  | M3/h |  |  |  |
| Electrical motor, each |  |  |  |  |  |  |  |  |  |  |  |  |  | kW |  |  |  |
| Pressure head |  |  |  |  |  |  |  |  |  |  |  |  |  | bar |  |  |  |
| Control panel |  |  |  |  |  |  |  |  |  |  |  |  |  | set |  |  |  |
| Common base frame of steel |  |  |  |  |  |  |  |  |  |  |  |  |  | set |  |  |  |
| Valve and connection pipe |  |  |  |  |  |  |  |  |  |  |  |  |  | set |  |  |  |
| A.2.1.2.1 L.F.O. transfer module (U632), including |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Electrical motor driven pump |  |  |  |  |  |  |  |  |  |  |  |  |  | set |  |  |  |
| Flow rate |  |  |  |  |  |  |  |  |  |  |  |  |  | M3/h |  |  |  |
| Electrical motor, each |  |  |  |  |  |  |  |  |  |  |  |  |  | kW |  |  |  |
| Pressure head |  |  |  |  |  |  |  |  |  |  |  |  |  | bar |  |  |  |
| Control panel |  |  |  |  |  |  |  |  |  |  |  |  |  | set |  |  |  |
| Common base frame of steel |  |  |  |  |  |  |  |  |  |  |  |  |  | set |  |  |  |
| Valve and connection pipe |  |  |  |  |  |  |  |  |  |  |  |  |  | set |  |  |  |
| A.2.2 LUBRICATING OIL SYSTEM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SYSTEM COMPOSITION. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.2.2.0 Lubricating oil storage tank (V731), including： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Capacity |  |  |  |  |  |  |  |  |  |  | M3 |  |  |  |  |  |  |
| High and low level alarm |  |  |  |  |  |  |  |  |  |  | set |  |  |  |  |  |  |
| Locally mounted level indicator |  |  |  |  |  |  |  |  |  |  | set |  |  |  |  |  |  |
| Steam heater 7 bars |  |  |  |  |  |  |  |  |  |  | set |  |  |  |  |  |  |
| Type of tank |  |  |  |  |  |  |  |  |  |  | Horizontal |  |  |  |  |  |  |
| A.2.2.1 Dirty lubricating oil storage tank (V734), including: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Capacity |  |  |  |  |  |  |  |  |  |  | M3 |  |  |  |  |  |  |
| High and low level alarm |  |  |  |  |  |  |  |  |  |  | set |  |  |  |  |  |  |
| Locally mounted level indicator |  |  |  |  |  |  |  |  |  |  | set |  |  |  |  |  |  |
| Steam heater 7 bars |  |  |  |  |  |  |  |  |  |  | set |  |  |  |  |  |  |
| Type of tank |  |  |  |  |  |  |  |  |  |  | Horizontal |  |  |  |  |  |  |
| A.2.2.2 Combined HFO and lub-oil separator and booster module (U690): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.2.2.2.0 HFO separator system： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ․Automatic fuel oil separator suitable for cleaning heavy fuel oil (max s.g: 0.991). Separator equipped with a built-on supply pump and built-in centripetal discharge pump. Change over valve system, futher complete with electric motor. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ․One steam heater, complete with safety valve and temperature regulator. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ․Capacity 4,000 lits/hr at 380 cSt. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.2.2.2.1 Lubricating oil separator system: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ․Automatic lubricating oil separator suitable for cleaning lubricating oil (max s.g: 0.991). Separator equipped with a built-on supply pump and built-in centripetal discharge pump. Change over valve system, futher complete with electric motor. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ․One steam heater, complete with safety valve and temperature regulator. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ․Capacity 4,800 lits/hr. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.2.2.2.2 Heavy fuel oil booster module: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Automatic filter. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ․Bypass filter (stand by). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ․Mixing tank. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ․Two circulation pumps. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ․Two steam heaters. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ․Viscosity control unit. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ․Control panel complete with main switch and all control functions. Including relays, switches and lamps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.2.3 COOLING SYSTEM: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SYSTEM COMPOSITION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.2.3.0 Cooling tower system, including： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cooling tower with centrifugal fans |  |  |  |  |  |  |  |  |  |  |  |  |  |  | set |  |  |
| Capacity |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Cal |  |  |
| Wet bulb temperature |  |  |  |  |  |  |  |  |  |  |  |  |  |  | °C |  |  |
| Water outlet temperature |  |  |  |  |  |  |  |  |  |  |  |  |  |  | °C |  |  |
| Max water inlet temperature |  |  |  |  |  |  |  |  |  |  |  |  | °C |  |  |  |  |
| Electrical power, each |  |  |  |  |  |  |  |  |  |  |  |  | KW |  |  |  |  |
| Water pump unit with electrical motor Driven |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | set |
| Flow rate |  |  |  |  | M3/h |  |  |  |  |  |  |  |  |  |  |  |  |
| Electrical motor |  |  |  |  | KW |  |  |  |  |  |  |  |  |  |  |  |  |
| Control cabinet |  |  |  |  | set |  |  |  |  |  |  |  |  |  |  |  |  |
| A.2.3.1 Pre-heating module: The pre-heating unit is used for the heating of H.T cooling water when starting the engine in cold condition. Complete with electric driven pump. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.2.3.2 HT cooling water heat exchanger: HT cooling water heat exchanger for recooling of the high temperature water. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.2.3.3 LT cooling water heat exchanger: LT cooling water heat exchanger for recooling of the low temperature water. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.2.3.4 Heat exchangers: the heat exchanger is used to transfer the heat available from the engine high temperature system to the hot water system. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Air compressor electric driven 11 kW. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Capacity 34M3/h |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Working pressure 30 bars. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Compressed air vessel. Capacity 1,500 lits working pressure 30bar complete with. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ․inlet outlet valves. ․Safety valve. ․Pressure gauge. ․Drain valve. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.2.5 Charge air system: A.2.5.0 Charge air filter: Air intake filter of the oil bath type. Intended to prevent the entering of sand, dust etc. into the engine air intake. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.2.5.1 Charge air silencer: air intake silencer, noise reduction 25 dB (A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.2.6 Exhaust gas system: A.2.6.0 Exhaust gas silencer: the exhaust gas silencer without spark arrestor, noise reduction 25 dB(A). Complete with counter flange |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.2.6.1 Exhaust gas ducting bellows: Corrosion resistant bellows for fitting between the ducting and exhaust gas silencer. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.2.6.2 Exhaust gas boiler: The exhaust gas boiler can produce the maximum amount of steam possible |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ․ Saturated steam production 3,600 kg/hr at 10 bar ․Steam generation at 170 0C ․The exhaust gas boiler system include the following: ․Exhaust gas boiler for mounting in the exhaust gas ducting. ․Evaporator coils with flanges. ․Soot blowing equipment. ․Standard assortment of valves, safety valves. ․Steam drum module including: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ○ Blow down drains ○ Accumulator ○ Circulating pump ○ Safety valve and control valves ○ Control board. Feed water module, including: ․Feed water pump ․Feed water tank ․Chemical treatment dosing unit ․Local control board |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A.2.7 Electric system: A.2.7.0 Main switchgear: - 1 common part: + Three-phase busbar system |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| • Operating voltage • Rated current • Rated voltage • Rated short time current |  |  |  |  |  |  |  |  | KV AMPS KV KV/1SEC. |  |  |  |  |  |  |  |  |
| +2 Generator Cubicles: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 triple-pole circuit breaker: vacuum draw out type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| • Operating volt • Rated current |  |  |  | KV AMPS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 station transformer cubicle (only high voltage): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| •One triple-pole loadbreaker switch |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| • Operating voltage • Three high volt fuses |  |  |  |  |  |  | KV AMPS |  |  |  |  |  |  |  |  |  |  |
| Outgoing feeder cubicle: One triple-pole circuit breaker: vacuum draw out type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operating voltage Rated current |  |  |  |  |  | KV AMPS |  |  |  |  |  |  |  |  |  |  |  |
| A.2.7.1 Neutral earthing board: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -1 neutral earthing boards, each consisting of: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +Isolating switch, single pole, load break type, with mechanical interlock. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| + One neutral earthing resistor, non-corrosive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |